MUSHAM VARUN - 2210110406

KARTHIK RAJ R - 2210110344

PAVAN KARTHIKEYA - 2210110281

REVANTH KAVURI – 2210110504

KANTIPUDI BHAVANA - 2210110342

EMBEDDED SYSTEMS HARDWARE

GROUP-2

TEAM MEMBERS

COURSE FACULTY

DR. ROHIT SINGH

KEYPAD BASED DOOR LOCK SYSTEM

PROJECT REPORT

Abstract

In this project, we plan to design a keypad-based door lock mechanism using a STM32 Development Board. In this mechanism, the microcontroller acts as the brain by managing the user input, pin verification and locking/unlocking mechanism. This project improves security by including a wrong code limit and remote pin reset option. This microcontroller, known for its processing capabilities, ensures seamless operation while maintaining a high level of security.

Table of Contents

| S. No | Topic | Pg. No |
| --- | --- | --- |
| 1. | Hardware and Software Requirements. | 4 |
| 2. | STM32 Development Board | 5,6 |
| 3. | 20x4 LCD | 7 |
| 4. | 4x4 Keypad | 8 |
| 5. | Bolt Wi-Fi module | 9 |
| 7. | Integrating the components | 12 |
| 8. | Code snippet | 13-20 |
| 9. | Benefits | 21,22 |
| 10. | Challenges | 23 |
| 11. | Future Iterations | 24,25 |
| 12. | References | 26 |
| 13. | Acknowledgements | 27 |

Hardware and Software Requirements

| Hardware Requirements | Software requirements |
| --- | --- |
| STM32F303RE | STM 32 Cube IDE |
| 20x4 LCD | Bolt IoT App |
| 4x4 Keypad |  |
| Bolt Wi-Fi Module |  |
| Buzzer |  |
| LEDs |  |

STM32 Microcontroller

The STM32 is a family of 32-bit microcontrollers based on the ARM Cortex-M processor core.

Widely used in embedded systems, IoT devices, and other applications.

Here are some of the specifications of the STM32:

● Processor core: ARM Cortex-M4

● Clock speed: 72MHz

● Flash memory: from 512KB

● RAM: from 64KB

● Operating voltage: 2.0V to 3.6V

● Power consumption: varies depending on usage (usually less than 200 mA)

● Interfaces: UART, SPI, I2C, USB, CAN, ADC, DAC, DMA, RTC, etc.

● Operating temperature: -40°C to +85°C or -40°C to +105°C

● Security: hardware and software encryption, secure boot, memory protection, etc.

The STM32 can be programmed using various integrated development environments (IDEs) such as STM32CubeIDE, Keil μVision. It supports many communication protocols and can be used for a

wide range of applications such as motor control, industrial automation, medical devices, and

consumer electronics.

Overall, the STM32 is a versatile and powerful microcontroller that offers a wide range of features and capabilities.

STM32F303RE Nucleo Board                              STM32F303RE Pin Diagram

20x4 LCD

A 20x4 LCD display refers to a liquid crystal display with a grid of 20 columns and 4 rows of characters.

20x4 means the display has 20 columns and 4 rows, resulting in a total of 80 characters (20 columns * 4 rows).

It is commonly used in embedded systems and electronics projects for displaying information like sensor readings, status messages, or menu options. The display is typically controlled through a set of commands sent via a microcontroller. Commands can

include clearing the display, positioning the cursor, and writing characters.

Throughout this project we will be using this LCD in 4-bit Mode (D4 to D7).

4x4 Keypad

The keypad serves as the primary user interface for entering access codes into the system. Interfacing with the STM32, the keypad provides a secure and user-friendly means of input. Its integration is crucial for enabling convenient access control, allowing users to input their access codes with ease.

Working Mechanism:

The microcontroller then scans the presence of a ‘key pressed’ event by setting the logic high on one of the outputs to the row while keeping other outputs on a low. This process is repeated for the next output continuously and ultimately loops back to the first output.

Simultaneously, the microcontroller checks for the input values that are connected to the columns. If there is at least one button being pressed, one of the column inputs changes from its default logic state. When an input is detected, the firmware refers to an internal input map to determine the exact physical button that is being pressed.

Bolt Wi-Fi Module

## General Specifications

| Parameters | Details |
| --- | --- |
| Connectivity and Processing Module | ESP8266 with custom firmware |
| MCU | 32-bit RISC CPU: Tensilica Xtensa LX106 |
| Power | 5V/1A DC via Micro-USB port or 5V and GND pins |
| Operating Voltage | 3.3V |
| CPU Clock Frequency | 80 MHz |
| MCU Internal Memory | 64 KB of instruction RAM; 96KB of data RAM |
| MCU External Memory | 4 MB Flash memory [QSPI] |
| GPIO pins | 5 Digital pins [3.3V logic] |
| ADC | 1 pin 10-bit ADC [0-1V input] |
| PWM | All 5 Digital pins capable of PWM [Software PWM] |
| Wi-Fi | 802.11 b/g/n (2.4 GHz Wi-Fi only) |
| UART | 8-N-1 3.3V TTL UART [using TX, RX,GND] [2400,4800,9600,19200 baudrate] |

WORKING

Keypad Based Door-Lock System operates seamlessly through a combination of hardware components and software functionalities, ensuring efficient access control and real-time monitoring.

Upon initialization, the STM32 board, functioning as the central control unit, establishes communication with the connected peripherals: the keypad, LCD and Wi-Fi Module. The keypad serves as the user interface, allowing individuals to input access codes securely. The GPIO pins on the STM32 facilitate this interaction, providing a responsive and reliable means of capturing user input.

In the event of an incorrect password attempt, the system is designed to trigger the Buzzer. The Buzzer beeps for 1 Second and Displays “Wrong Password, Try Again” LCD. The User can enter max of 3 incorrect pins. After the 3 trails, the buzzer beeps for 2 seconds and the user must wait for 10 or 20 seconds for trying again.

If number of wrong attempts reaches 9, then the system will enter lockdown mode. The only way to come out of this lockdown mode is through the Bolt IoT app.

If the user wants to change the pin, then he needs to go to the Bolt IoT app and set the wi-fi enabled GPIO pin high. Then Logic High will be read by the STM32 board and enters the pin change mode.

The collaborative operation of these components results in a robust Smart Door Security System that enhances access control. The integration of hardware and software ensures a seamless and responsive user experience, contributing to the overall security and smart management of access control in the context of the Internet of Things (IoT).

CODE

Header Files Used:

<LCD2004.h>:

Used to control the 20x4 LCD. The functions used from this header are:

- lcd_init(): Initialise the LCD

- lcd_put_cur(row,column): Put the cursor at a particular position

- lcd_clear(): Clear the LCD screen

- lcd_send_string(string): Send a string to display on LCD

<Keypad.h>:

Used to get input from the 4x4 Keypad.

<stdlib.h>

Used to dynamically allocate memory to a certain variable.

<string.h>

Used to perform basic operations like size of string, compare two strings.

Variables Used

Key: Used to store the input of keypad

User[]: Used to store the pin entered by user.

Pass[]: Used to store the correct pin.

i: Used as a position pointer

trail: Used to count number of wrong attempts

lockdown: Used to count number of wrong attempts and later used to implement lockdown mode

mul: Counts number of minutes the system is in lockdown mode.

Initializing the system:

Upon turning on the STM, the following operations are done:

- Start the timer

- Set pin PA4 to reset state

- Empty the user[] array

- Initialize the LCD

- Put the Cursor at position 0,0

- Print “Enter Pin” on LCD

Taking the Input:

After the initialization process, the system waits for the input from keypad. Until the user enter the “D” key, the system will be taking input from the user. The “*” key acts as Backspace. We have given a delay of 250ms after each input to avoid multiple characters being entered on one press.

Comparing the Input:

After pressing the “D” key, the STM compares the user input pin with the correct pin using strcmp.

strcmp returns 0 if the strings are same, else returns -1/1.

Correct User Pin:

If the user enters the correct pin, then the LCD displays “Welcome” and the Green LED will be on for 3 seconds. Then the STM will wait for the user to enter the “D” key. Once the user enters the “D” key, the system will go back to the stage of Pin input.

Wrong Pin:

If the User enter wrong pin, then the STM will first increment the “trail” variable by 1, then checks if the value of trail is less than 3 or not.

If trail<3:

If the number of trails is less than 3 (starts from 0), then the buzzer will beep, and the blue LED will be ON for 500ms. During this time the LCD will print “Incorrect Pin, Try Again”. Then the STM will go back to taking the user input.

If trail==3 and lockdown<3:

If number of trails are 3, the lockdown variable will be incremented by 1 and buzzer will beep 3 times, 500ms each. Then the user will be asked to wait for 10*(value of lockdown variable) seconds. After this the STM will go back to taking the user input.

If lockdown==3:

If the value of lockdown is 3, which means that the user has input wrong pin 9 times, then the buzzer will beep 3 times, 500ms each. And our setup will go into lockdown mode. In this mode, the Blue and Green LED will stay ON till the lockdown mode is turned off and the user won’t be able to enter anything using the keypad. The LCD will display the since when the system is in lockdown mode. To turn off the Lockdown mode, the user needs to turn on the Wi-fi Enabled Pin on the Bolt IoT Wi-fi Module using the Bolt IoT app.

Reset Mode:

If the user wants to reset the Pin, then the user is required to turn on the Wi-fi Enabled Pin on the Bolt IoT Wi-fi Module using the Bolt IoT app. Upon setting this Pin to high, the setup will enter Pin Reset mode. The user is then required to enter a new 5-digit Pin. Once the user enters the 5-digit pin, the system will update the pass[] variable.

BENEFITS

The keypad-based door lock mechanism using an STM32 Development Board offers several benefits:

- Enhanced Security:

  - The project introduces an added layer of security with pin verification and a wrong code limit. This helps prevent unauthorized access and enhances the overall security of the door lock system.

- User-Friendly Interface:

  - The keypad interface provides a user-friendly and familiar method for input, making it convenient for users to interact with the door lock system.

- Seamless Operation:

  - Leveraging the processing capabilities of the STM32 microcontroller ensures smooth and seamless operation of the door lock mechanism. The microcontroller efficiently manages user inputs and implements security features without compromising performance.

- Flexibility in Code Implementation:

  - The use of STM32 microcontroller and its associated development environment provides flexibility in coding, allowing for the implementation of custom features and enhancements as needed for the door lock system.

- Reliability and Durability:

  - The STM32 microcontroller is known for its reliability and robust performance. Implementing the door lock mechanism with this microcontroller ensures a durable and dependable solution.

- Real-time Operation:

  - The microcontroller's real-time processing capabilities enable timely responses to user inputs and security events. This is crucial for the effectiveness of a door lock system.

- Low Power Consumption:

  - Efficient power management features of the STM32 microcontroller contribute to lower power consumption, making the door lock system energy-efficient and suitable for long-term use.

- Remote Pin Reset:

  - The only way to reset the Pin (from user side) is through the Bolt IoT app.

- Lock-Down Mode:

  - After too many wrong attempts, our system will go into Lockdown mode and no one can interface with the system until and unless the user turns it off from the Bolt IoT app.

CHALLENGES

Difficulty in Code Modification:

The need for code modification to change the keypad password may be considered a drawback, especially if a more user-friendly and accessible mechanism for password changes is desired.

Limited Connectivity Options:

The STM32 microcontroller may have limitations in terms of connectivity options. Integrating additional features or connecting the door lock system to external devices might be constrained by the available interfaces.

Security Concerns with Keypad Input:

Keypad-based systems are susceptible to unauthorized access through methods like code guessing or eavesdropping. Implementing advanced security measures, such as anti-spoofing mechanisms, could be challenging with a keypad-only interface.

Limited User Capacity:

The STM32 Development Board may have limited storage capacity for user data. This limitation could restrict the number of users the system can accommodate, making it less suitable for applications requiring a large user base.

FUTURE ITERATIONS

- Biometric Integration:

  - Integrate biometric authentication methods, such as fingerprint or facial recognition, to enhance security and provide a more user-friendly and modern authentication experience.

- Smart Card or RFID Integration:

  - Add support for smart card or RFID-based authentication, offering users alternative and convenient ways to access the door lock system.

- Wireless Connectivity:

  - Implement wireless connectivity options, such as Wi-Fi or Bluetooth, to enable remote management, firmware updates, and real-time monitoring of the door lock system through a dedicated mobile app or web interface.

- Mobile App Integration:

  - Develop a mobile application that allows users to control and monitor the door lock system remotely. This app could provide features like user management, access logs, and instant alerts.

- Advanced User Management:

  - Enhance user management capabilities by allowing administrators to easily add or remove users, assign different access levels, and modify user permissions through a user-friendly interface.

- Dynamic PIN Length Configuration:

  - Provide users with the ability to dynamically configure the length of their PINs based on their security preferences, offering a customizable and adaptable security solution.

- Integration with Home Automation Systems:

  - Integrate the door lock mechanism with popular home automation systems, allowing users to include it in their smart home ecosystems and control it alongside other connected devices.

- Voice Command Integration:

  - Incorporate voice recognition technology to enable users to unlock the door using spoken commands, adding a hands-free and user-friendly access method.

- Integration with Emergency Services:

  - Establish integration with emergency services or security monitoring systems to automatically notify authorities in case of a security breach or emergency.

REFERENCES

LCD interfacing: https://www.youtube.com/watch?v=ITTBWSQTi3c

Keypad Interfacing: https://www.youtube.com/watch?v=sqf74jHEKn4

Mastering STM32, a step-by-step guide by Carmine Noviello

ACKNOWLEDGEMENT

We are immensely grateful to the university and Dr. Rohit Singh for offering this course. The project has been an incredible opportunity for us to learn and grow, both academically and professionally. We have been challenged and inspired and we have had the opportunity to apply our knowledge and skills to real-world problems.

We believe that this course and project have prepared us well for our future careers. From this project we have developed a solution-oriented approach which would add a great value for our future. We have learned valuable skills such as critical thinking, problem-solving, and teamwork. It also provided us with practical experience in microcontroller programming, embedded systems, and security implementation.